Thermodynamic characterization of the marine boundary layer during fog and dew events in the hyperarid coastal climate of the Atacama Desert

F.M. Narbona,   S. Acevedo, A. C. Del Río, Siegmund, F. Lobos-Roco,

2026-08-30

Introduction

In the coastal Atacama Desert, precipitation is almost null (Houston, 2006); thus, fog and dew are the main water sources. These water sources sustain fragile hyperarid ecosystems such as Tillandsia landbeckii and potentially provide a freshwater supply to local communities (Lobos-Roco et al., 2024; Río et al., 2021).

Fog and dew are condensation processes that ocurrs within the marine boundary layer (MBL), governed by turbulent mixing (fog), advection and thermal stratification (dew) (Lobos-Roco et al., 2024; Stull, 1988). Fog forms when a moist air parcel saturates, then reaching the dew point (Tair ≤ Tdew) (Gultepe et al., 2007). In northern Chile, fog results from the advection of marine stratocumulus clouds, which are formed over the South East Pacific (SEP) Ocean under high subsidence promoted by the South Pacific Anticyclone (Cereceda et al., 2002; Cereceda et al., 2008; Rutllant & Montecino, 2002). This subsidence contrasts with the cold humboldt current and the cold water upwelling forming a strong thermal inversion (~20 K) that limits vertical mixing, driving the straticumulus cloud formation. The anticyclonic circulation in the SEP transports these well-formed strstocumulus up to 50–60 km inland (Lobos-Roco et al., 2024; Río et al., 2021). One of the main thermodynamic characteristic of stratocumulus cloud-fog in the Atacama is that it develops under a well-mixed MBL, with low potential-temperature gradients (∂θ/∂z < 3.1×10⁻³ K m⁻¹) (Lobos Roco et al., 2018) and continuous positive moisture advection (∂q/∂t ≈ +0.016 g kg⁻¹ h⁻¹; Lobos-Roco et al., 2024).

Dew forms over surfaces, which driven by nocturnal radiative cooling, reach the dew point (Tsurf ≤ Tdew), condensing directly on solid surfaces under clear, calm, humid conditions (Monteith, 1957). In terms of thermodynamcis, dew forms under a more stable surface layer (∂θ/∂z ≈ 3.8×10⁻³ K m⁻¹) and a net negative vapour tendency (∂q/∂t ≈ −0.25 g kg⁻¹ h⁻¹) (Lobos-Roco et al., 2024). This contrast shows that ∂θ/∂z is a reliable variable to distinguish between fog and dew, unlike the moisture gradient, which is limited by MBL humidity homogeneity (Lobos-Roco et al., 2024). Contrary, since dew fromation reduces the air humidity by transfering it to the surface, the ∂q/∂t also might play a role in distinguish dew from fog.

Fog and dew can be quantified by measuring their respective collection rates. For that porpuse, standardised passive collectors have been developed. On the one hand, the Standard Fog Collector (SFC), collects fog droplets over a 1 m² polypropylene mesh (Schemenauer & Cereceda, 1994). In the Atacama, fog collects rates from 1 to 7 L m-2 day-1. On the other hand, the Standard Dew Collector (SDC), collects dew by cooling a plate surface of a 1 m² by nocturnal radiative loss (Muselli et al., 2002). In the Atacama, collection rates of dew are in the oder of XXX. Despite the effort of quantify fog and dew through the passive collection, they fail to distinguish between the two phenomena due to an instrumental overlap in their collection periods, which leaves their relative contribution highly uncertain (Lobos-Roco et al., 2024).

In hyperarid regions, where fog and dew constitute the primary water inputs sustaining ecosystems, accurately measuring their dynamics is essential to understanding their contribution to the water and carbon balance of these systems (Lobos-Roco et al., 2024; Wang et al., 2016). Both processes supply moisture that biological soil crusts and other surface-dwelling organisms rely on, directly influencing their carbon fixation capacity, which underscores the need for methods that properly constrain the individual contribution of each phenomenon (Chamizo et al., 2021; Li et al., 2025). In contrast to fog, our understanding of dew remains very limited (Li et al., 2025), and resolving this ambiguity is important not only to understand hyperarid ecosystem hydrology, but also to develop improved collection technologies and inform water-related public policy in arid regions.

In the context of the highly uncertainty that fog and dew collectors produces, in this study we aim to answer how do atmospheric stability and humidity tendency differ between fog and dew eventsin the coastal Atacama Desert? We hypothesise that dew events are systematically underestimated by standard collectors. Likewise fog and dew show marked difference in the atmospheric stability and mositure tendency. We test three falsifiable hypotheses. Firstly, (occurrence): the frequency of dew events compared to total events exceeds the ones recorded by SDC. Secondly (structure): fog forms under a well-mixed marine boundary layer (lower ∂θ/∂z), dew under a more stratified layer (higher ∂θ/∂z), while ∂q/∂z and ∂U/∂z do not discriminate between regimes, as both require a near-saturated, well-mixed vapour column. Thirdly (dynamics): fog is marked by continuous moisture advection (∂q/∂t > 0), while dew by a surface moisture sink (∂q/∂t ≤ 0), reflecting opposite atmospheric water vapor tendency. To test these hypethesis, our methodology was based a combination of in-situ meteorological observations from a topohgraphic transect and satellite information retieved from GOES satellite. Thise manuscript is structured as follows....

References

Cereceda, P., Larrain, H., Osses, P., Farías, M., & Egaña, I. (2008). The spatial and temporal variability of fog and its relation to fog oases in the atacama desert, chile. Atmospheric Research, 87(3-4), 312–323. https://doi.org/10.1016/j.atmosres.2007.11.012

Cereceda, P., Osses, P., Larrain, H., Farı́as, M., Lagos, M., Pinto, R., & Schemenauer, R. S. (2002). Advective, orographic and radiation fog in the tarapacá region, chile. Atmospheric Research, 64(1-4), 261–271. https://doi.org/10.1016/s0169-8095(02)00097-2

Chamizo, S., Rodríguez-Caballero, E., Moro, M. J., & Cantón, Y. (2021). Non-rainfall water inputs: A key water source for biocrust carbon fixation. Science of The Total Environment, 792, 148299. https://doi.org/10.1016/j.scitotenv.2021.148299

Gultepe, I., Tardif, R., Michaelides, S. C., Cermak, J., Bott, A., Bendix, J., Müller, M. D., Pagowski, M., Hansen, B., Ellrod, G., Jacobs, W., Toth, G., & Cober, S. G. (2007). Fog research: A review of past achievements and future perspectives. In Fog and boundary layer clouds: Fog visibility and forecasting (pp. 1121–1159). Birkhäuser Basel. https://doi.org/10.1007/978-3-7643-8419-7_3

Houston, J. (2006). Variability of precipitation in the Atacama Desert: Its causes and hydrological impact. International Journal of Climatology, 26(15), 2181–2198. https://doi.org/10.1002/joc.1359

Keim-Vera, K., Lobos-Roco, F., Aguirre, I., Merino, C., & Río, C. del. (2024). Fog types frequency and their collectable water potential in the atacama desert. Atmospheric Research, 312, 107747. https://doi.org/10.1016/j.atmosres.2024.107747

Li, H., Han, C., Yang, Y., & Chen, R. (2025). Advances and challenges in dew research on land surface: A review. Hydrology, 12(12), 320. https://doi.org/10.3390/hydrology12120320

Lobos Roco, F., Vilà-Guerau de Arellano, J., & Pedruzo-Bagazgoitia, X. (2018). Characterizing the influence of the marine stratocumulus cloud on the land fog at the atacama desert. Atmospheric Research, 214, 109–120. https://doi.org/10.1016/j.atmosres.2018.07.009

Lobos-Roco, F., Suárez, F., Aguirre-Correa, F., Keim, K., Aguirre, I., Vargas, C., Abarca, F., Ramírez, C., Escobar, R., Osses, P., & Río, C. del. (2024). Understanding inland fog and dew dynamics for assessing potential non-rainfall water use in the atacama. Journal of Arid Environments, 221, 105125. https://doi.org/10.1016/j.jaridenv.2024.105125

Monteith, J. L. (1957). Dew. Quarterly Journal of the Royal Meteorological Society, 83(357), 322–341. https://doi.org/10.1002/qj.49708335706

Muselli, M., Beysens, D., Marcillat, J., Milimouk, I., Nilsson, T., & Louche, A. (2002). Dew water collector for potable water in ajaccio (corsica island, france). Atmospheric Research, 64(1-4), 297–312. https://doi.org/10.1016/s0169-8095(02)00100-x

Río, C. del, Lobos-Roco, F., Latorre, C., Koch, M. A., García, J.-L., Osses, P., Lambert, F., Alfaro, F., & Siegmund, A. (2021). Spatial distribution and interannual variability of coastal fog and low clouds cover in the hyperarid atacama desert and implications for past and present tillandsia landbeckii ecosystems. Plant Systematics and Evolution, 307(5). https://doi.org/10.1007/s00606-021-01782-z

Rutllant, J., & Montecino, V. (2002). Multiscale upwelling forcing cycles and biological response off north-central chile. Revista Chilena de Historia Natural, 75(1). https://doi.org/10.4067/s0716-078x2002000100020

Schemenauer, R. S., & Cereceda, P. (1994). A proposed standard fog collector for use in high-elevation regions. Journal of Applied Meteorology, 33(11), 1313–1322. https://doi.org/10.1175/1520-0450(1994)033<1313:apsfcf>2.0.co;2

Stull, R. B. (1988). An introduction to boundary layer meteorology. Kluwer Academic Publishers. https://doi.org/10.1007/978-94-009-3027-8

Wang, L., Kaseke, K. F., & Seely, M. K. (2016). Effects of non‐rainfall water inputs on ecosystem functions. WIREs Water, 4(1). https://doi.org/10.1002/wat2.1179